EE092IU

DIGITAL SIGNAL PROCESSING LABORATORY

Lab 4

# Z TRANSFORM

# Full name: Nguyen Minh Duc

Student number: ITITIU21045

Class: DSP Lab Tuesday afternoon

Date: April 11th, 2023

- OBJECTIVES

Students know how to write m-file in Matlab to illustrate the properties of Z transform.

# REQUIRED EQUIPMENT

  - Computer

  - Matlab software

# INTRODUCTION

Just as with the Laplace transform for continuous-time signals and systems, the Z-transform provides a way to represent discrete-time signals and systems, and to process discrete-time signals. Although the Z-transform can be related to the Laplace transform, the relation is operationally not very useful. However, it can be used to show that the complex z-plane is in a polar form where the radius is a damping factor and the angle corresponds to the discrete frequency w in radians. Thus, the unit circle in the z-plane is analogous to the jw axis in the Laplace plane, and the inside of the unit circle is analogous to the left-hand s-plane. We will see that once the connection between the Laplace plane and the z-plane is established, the signiﬁcance of poles and zeros in the z-plane can be obtained like in the Laplace plane.

The representation of discrete-time signals by the Z-transform is very intuitive—it converts a sequence of samples into a polynomial. The inverse Z-transform can be achieved by many more methods than the inverse Laplace transform, but the partial fraction expansion is still the most commonly used method. Using the one-sided Z-transform, for solving difference equations that could result from the discretization of differential equations, but not exclusively, is an important application of the Z-transform.

## Mathematical Definition

The 𝑧 transform of a discrete time signal is defined as:

| ∞ 𝑋(𝑧) = ∑ 𝑥[𝑛]𝑧−𝑛 𝑛=−∞ | (1) |
| --- | --- |

Example 1: Compute the z-transform of the sequence

xn  3,5, 4,3, 0  n  3

An alternative and more elegant computation of the z-transform

Example 2: Compute the z-transform of the sequence

x n  0.9nu n

In MATLAB, the z-transform

F  z

of a sequence

f n

is computed easily by using the

command ztrans. Moreover, the inverse z-transform of a function the command iztrans.

F  z is computed by using

Before using these two commands, the declaration of the complex variable z and of the discrete time n as symbolic variables is necessary. Recall that in order to deﬁne a symbolic variable, the command syms is used.

Example 3: Compute the z-transform of the sequence

f n  2n

z

Example 4: Compute the inverse z-transform of the function

F  z  

 z  2

Example 5: Using Partial Fraction Expansion of a Rational Function, express in the partial fraction form the signal which in the z-domain is given by

 

z2  3z 1

X  z   z3  5z2  2z  8

Example 6: Using Partial Fraction Expansion of a Rational Function, express in the partial fraction form the signal which in the z-domain is given by

 

z2  3z 1

X   z   z3  3z  2

We also can use residue command

## Basic Properties of the Z Transform

Table 1: Z-transform property

## Z Transform Pairs

Table 2: Z-transform pairs

## 3.4 Using the z-Transform to Solve Difference Equations

Example 7: Find the solutions of the difference equation y[n] +0.5y[n-1] + 2y[n-2] = 0.9n u[n], where y[n] = 0, n < 0

## Solution:

We have:

The computational procedure as follow:

# In Matlab:

In order to confirm that y[n] is in fact the solution of the difference equation, y[n] is inserted to the difference equation, if it satisfies the difference equation, then it is indeed its solution

# PROCEDURE

## Problem 1:

Express in partial fraction form the signal and determine

x n

X  z  

12  38z1 11z2  3z3  54z4

1 5z1  6z2

2z2  z 1

X  z  

z3  3z  2

X  z  

5 11z1

1	2

## Problem 2:

1 5z	 6z

Compute of the Z-transform of the following sequence using the symbolic toolbox of MATLAB

ℎ1[𝑛] = 0.8𝑢(𝑛)

ℎ2[𝑛] = 𝑢[𝑛] − 𝑢[𝑛 − 10] ℎ3[𝑛] = 𝑐𝑜𝑠(𝜔0𝑛)𝑢[𝑛]

ℎ4[𝑛] = ℎ1[𝑛]ℎ3[𝑛]

clc

clear all

pkg load symbolic

syms n z w

x=0.8;

h_1 = symsum(x.*(z.^-n),n,0,inf)

h_2 = symsum((z.^-n)-(z^(-n-10)),n,0,inf)

h_3 = symsum(cos(w*n)*(z.^-n),n,0,inf)

h_4 = symsum(x.*cos(w*n),n,0,inf)

disp('Sequence 1')

ztrans(h_1)

disp('Sequence 2')

ztrans(h_2)

disp('Sequence 3')

ztrans(h_3)

disp('Sequence 4')

ztrans(h_4)

## Problem 3:

Consider an FIR filter with impulse response

hn   n  n 1   n  2

Find the filter output for an input

x n  cos 2 n 3 u n  u n 14

Use the convolution sum to find the output, and verify your results with MATLAB. Use z-transform to find the output

We have: y(n) = x(n)*h(n) = k=-∞∞xk*h(n-k)

By convolution with impulse property

x(n)*δn-n0=xn-n0

Therefore, y(n) = x(n)*[ δn+δn-1+δn-2]

y(n) = x(n) + x(n – 1) + x(n – 2)

x(n) = cos(2πn3)*[un-u(n-14)]

Therefore, y(n) = cos(2πn3)*un-un-14+cos2πn-13*un-1-un-15+cos(2π(n-2)3)*[un-2-u(n-16)]

Code:

n1 = 0:1:13;

x1 = cos(2*(pi/3)*n1);

stem(n1, x1)

subplot(2,2,1)

n2 = 1:1:14;

x2 = cos(2*(pi/3)*(n2-1));

stem(n2, x2)

subplot(2,2,2)

n3 = 2:1:15;

x3 = cos(2*(pi/3)*(n3-2));

stem(n3, x3)

subplot(2,2,3)

n4 = n1:1:n3;

y = cos(2*(pi/3)*n1)+cos(2*(pi/3)*n1)+cos(2*(pi/3)*(n2-1))+cos(2*(pi/3)*(n3-2));

stem(n1, y)

subplot(2,2,4)

## Problem 4:

Use z-transform to ﬁnd the solution of the difference equations:

a/ y[n] + 1.5 y[n-1] + 0.5y[n-2] = x[n] + x[n-1], where x[n] = 0.8n u[n]

b/ y[n] – y[n-1] = x[n] + x[n-1], where x[n] = 0.8n u[n]

Plot the solution for 0 < n < 20.

Conﬁrm your result by inserting the obtained solution in the difference equation.

a/ There is no initial condition, we take z-transform on both sides:

Y(z) + 1.5*z-1Y(z) + 0.5*z-2Y(z) = X(z) + z-1X(z)

Y(z)*(1 + 1.5*z-1 + 0.5*z-2) = X(z)*(1+z-1)

Y(z)X(z)=Hz=1+z-11+1.5*z-1+0.5*z-2=z(z+1)z2+1.5*z+0.5 (1)

We have: x(n) = 0.8nu[n]

- X(z) = zz-0.8

From equation (1), we have:

Y(z) = z(z+1)z2+1.5*z+0.5*zz-0.8=z(z+1)z-0.5*(z-1)*zz-0.8

Using partial fraction:

Y(z)z=z(z+1)z-0.5*z-1*(z-0.8)=Az-0.5+Bz-0.8+C(z-1)

So A=limz→0.5z(z+1)z-1*(z-0.8)=5

B=limz→0.8z(z+1)z-0.5*(z-1)=-24

C=limz→1z(z+1)z-0.5*(z-0.8)=20

Therefore, Y(z)z=5z-0.5+-24z-0.8+20(z-1)

Y(z)=5zz-0.5+-24zz-0.8+20z(z-1)

Using inverse Z-transform:

Yz=5(0.5)nu[n] - 24(0.8)nu[n] + 20u[n]

Yz=[5(0.5)n - 24(0.8)n + 20]*u[n]

Code:

n=0:20;

yn = ((5*(0.5).^n)-(24*(0.8).^n)+(20))

stem(n, yn)

grid on

xlabel("Samples (n)")

ylabel("Magnitude")

b/ There is no initial condition, we take z-transform on both sides:

Y(z) + z-1Y(z) = X(z) + z-1X(z)

Y(z)*(1 + z-1) = X(z)*(1 + z-1)

Y(z)X(z)=Hz=1+z-11+z-1=1 (1)

We have: x(n) = 0.8nu[n]

- X(z) = zz-0.8

From equation (1), we have:

Y(z) = 1*zz-0.8=zz-0.8

Using inverse Z-transform:

Yz= 0.8nu[n]

Code:

n=0:20;

yn = (0.8).^n

stem(n, yn)

grid on

xlabel("Samples (n)")

ylabel("Magnitude")